# Twos’ Complement and Boolean Operators

### Two’s Complement

Two’s Complement Representation: Negative integers use two’s-complement representation, (If you add a number to its additive inverse, the sum is zero.)

Two’s-complement representation is useful to processor designers because it removes the need for separate digital circuits to handle both addition and subtraction. For example, if presented with the expression A – B, the processor can simply convert it to an addition expression: A + (–B).

Using the 8-bit binary value 00000001, for example, its two’s complement turns out to be 11111111, as can be seen as follows:

- What is the two’s complement for 01111011?

10000100 + 1 = 10000101

- What is the two’s complement for 01010101?

10101010 + 1 = 10101011

### Two’s Complement for Hexadecimal Values

Hexadecimal Two’s Complement

To create the two’s complement of a hexadecimal integer, reverse all bits and add 1. An easy way to reverse the bits of a hexadecimal digit is to subtract the digit from 15. Here are examples of hexadecimal integers converted to their two’s complements:

6A3D  6-15 is 9, A or 10 -15 is 5, 3-15 is 12 or C, D or 13 – 15 is 2

- 95C2

- Then add 1  95C3

- Find the two’s complement for 118E16.

EE71 + 1 = EE72

- Find the two’s complement 760B216.

### Converting Signed Decimal to Hexadecimal

To convert a signed decimal integer to hexadecimal, do the following:

Step 1: Convert the absolute value of the decimal integer to hexadecimal.

Step 2: If the decimal integer was negative, create the two’s complement of the hexadecimal number from the previous step.

- Convert decimal -611 to hexadecimal.

-263 = D9C + 1 = D9D

- Convert decimal -2885 to hexadecimal.

-B45 = 4BA + 1 = 4BB

### Converting Signed Hexadecimal to Decimal

To convert a signed hexadecimal integer to decimal, do the following:

Step 1: If the hexadecimal integer is negative, create its two’s complement; otherwise, retain the integer as is. Using the integer from the previous step, convert it to decimal.

Step 2: If the original value was negative, attach a minus sign to the beginning of the decimal integer.

- Convert -52A16 to a decimal value.

-52A = AD5 + 1 = -AD6 = -2774

- Convert -604E16 to a decimal value.

9FB1 + 1 = 9FB2 = -40882

Example:

0 0 1 1 1 1 0 1

⌐1 1 0 0 0 0 1 0

- Find the NOT expression of 10101101.

01010010

- Find the NOT expression of 11111011.

00000100

| X | Y | X AND Y |
| --- | --- | --- |
| 0 | 0 | 0 |
| 0 | 1 | 0 |
| 1 | 0 | 0 |
| 1 | 1 | 1 |

- Solve:

1 0 1 1 1 0 1 1

AND	0 0 0 1 1 1 0 0

00011000

- Solve:

1 1 1 1 1 1 0 0

AND	0 0 0 1 0 1 1 0

000010100

| X | Y | X v Y |
| --- | --- | --- |
| 0 | 0 | 0 |
| 0 | 1 | 1 |
| 1 | 0 | 1 |
| 1 | 1 | 1 |

- Solve:

0 0 1 1 1 1 0 1

OR	1 1 1 1 1 1 0 1

111111101

- Solve:

1 0 0 1 1 1 1 0

OR	1 1 0 0 0 0 1 1

11011111